Carbon footprint report for HQ

01 April 2024 to 31 March 2025

HQ emitted 5,845 kgCO2e (Kilogrammes of carbon dioxide equivalent) for 2024/25 (across scope 1 and 2).  This can be presented as 6 tCO2e (tonnes of carbon dioxide equivalent) with an intensity indicator of 0.34 tCO2e per total full-time equivalent employee (FTE) and 0 tCO2e per million GBP £.

When Scope 3 is added, this brings the total to 42 tCO2e.

Table 1. UK GHG emissions and energy use data for period 01 April 2024 to 31 March 2025

| Emissions source | Units | kWh | Carbon (kgCO2e) | Carbon (tCO2e) |
| --- | --- | --- | --- | --- |
| Scope 1 |  |  |  |  |
| Large car (hybrid) | 1,250 km | 783 | 193.65 | 0.19 |
| Natural gas | 9,769 kWh | 9,769 | 1,786.66 | 1.79 |
| Total Scope 1 |  |  | 1,980 | 2 |
| Scope 2 |  |  |  |  |
| UK National Grid electricity | 18,668 kWh | 18,668 | 3,865.15 | 3.87 |
| Total Scope 1 & 2 |  |  | 5,845 | 6 |
| Total tCO2e per *FTE on gross scope 1 & 2 |  |  |  | 0.34 |
| Total tCO2e per *£m Turnover on gross scope 1 & 2 |  |  |  | 0 |

| Emissions source | Units | kWh | Carbon (kgCO2e) | Carbon (tCO2e) |
| --- | --- | --- | --- | --- |
| Scope 3 |  |  |  |  |
| Cat 01 - Purchased Goods & Services |  |  |  |  |
| By spend - By SIC emissions intensity - C - Manufacturing | £331 |  | 115.12 | 0.12 |
| By spend - By SIC emissions intensity - F - Construction | £9,352 |  | 769.85 | 0.77 |
| By spend - By SIC emissions intensity - G - Wholesale and retail trade; repair of motor vehicles and motorcycles | £9,762 |  | 535.75 | 0.54 |
| By spend - By SIC emissions intensity - H - Transport and storage | £108 |  | 115.63 | 0.12 |
| By spend - By SIC emissions intensity - I - Accommodation and food services | £924 |  | 59.19 | 0.06 |
| By spend - By SIC emissions intensity - J - Information and communication | £30,052 |  | 0.00 | 0.00 |
| By spend - By SIC emissions intensity - K - Financial and insurance activities | £13,636 |  | 0.00 | 0.00 |
| By spend - By SIC emissions intensity - N - Administrative and support service activities | £5,039 |  | 138.27 | 0.14 |
| By spend - By SIC emissions intensity - R - Arts, entertainment and recreation | £4,068 |  | 148.82 | 0.15 |
| By spend - By SIC emissions intensity - S - Other service activities | £722 |  | 13.20 | 0.01 |
| Material use (BEIS) - Water consumption - Water supply | 297 m3 |  | 45.47 | 0.05 |
| Supply Chain Dashboard |  |  | 0 | 0 |
| Cat 03 - Fuel & energy related activities |  |  |  |  |
| Transmission & distribution losses - T&D for UK national grid electricity | 20,135 kWh |  | 368.47 | 0.37 |
| Well-to-tank (WTT) - Business travel - Passenger Vehicles - WTT - Diesel - Medium car | 29,950 mile |  | 1,977.65 | 1.98 |
| Well-to-tank (WTT) - Business travel - Passenger Vehicles - WTT - Diesel - Small car | 2,912 mile |  | 159.76 | 0.16 |
| Well-to-tank (WTT) - Business travel - Passenger Vehicles - WTT - Hybrid - Large car | 4,462 mile |  | 284.45 | 0.28 |
| Well-to-tank (WTT) - Business travel - Passenger Vehicles - WTT - Petrol - Large car | 4,432 mile |  | 536.97 | 0.54 |
| Well-to-tank (WTT) - Business travel - Passenger Vehicles - WTT - Petrol - Medium car | 23,576 mile |  | 1,880.81 | 1.88 |
| Well-to-tank (WTT) - Business travel - Passenger Vehicles - WTT - Petrol - Small car | 18,390 mile |  | 1,188.26 | 1.19 |
| Well-to-tank (WTT) - Business travel - Travel - WTT - Local bus (not London) | 14,333 mile |  | 732.13 | 0.73 |
| Well-to-tank (WTT) - Business travel - Travel - WTT - National rail | 8,805 mile |  | 127.10 | 0.13 |
| Well-to-tank (WTT) - Business travel - Travel - WTT - Regular taxi | 1,112 mile |  | 92.61 | 0.09 |
| Well-to-tank (WTT) - Electricity - WTT- UK electricity (generation) | 20,135 kWh |  | 924.18 | 0.92 |
| Well-to-tank (WTT) - Electricity - WTT- UK electricity (T&D) | 18,668 kWh |  | 74.11 | 0.07 |
| Well-to-tank (WTT) - Fuels - WTT - Natural Gas (kWh) | 11,010 kWh |  | 332.61 | 0.33 |
| Cat 05 - Waste disposal |  |  |  |  |
| Commercial and industrial waste (Closed-loop) | 1,010 kg |  | 6.47 | 0.01 |
| Commercial and industrial waste (Combustion) | 1,010 kg |  | 6.47 | 0.01 |
| Water discharge - Water treatment | 317 m3 |  | 58.81 | 0.06 |
| Cat 06 - Business Travel |  |  |  |  |
| By mileage - Cars (by size) - Diesel - Medium | 3,355 mile | 3,583.87 | 907.47 | 0.91 |
| By mileage - Cars (by size) - Diesel - Small | 920 mile | 816.58 | 207.19 | 0.21 |
| By mileage - Cars (by size) - Petrol - Medium | 2,710 mile | 3,335.97 | 773.09 | 0.77 |
| By mileage - Public transport - National rail | 339 mile |  | 19.32 | 0.02 |
| By mileage - Taxis - Regular taxi | 1,112 mile |  | 372.26 | 0.37 |
| Cat 07 - Employee Commuting |  |  |  |  |
| Cars (by size) - Battery Electric - Large | 2,210 mile |  | 175.15 | 0.18 |
| Cars (by size) - Diesel - Medium | 23,057 mile |  | 6,236.62 | 6.24 |
| Cars (by size) - Diesel - Small | 1,992 mile |  | 448.62 | 0.45 |
| Cars (by size) - Hybrid - Average | 788 mile |  | 159.88 | 0.16 |
| Cars (by size) - Hybrid - Large | 3,685 mile |  | 918.44 | 0.92 |
| Cars (by size) - Petrol - Large | 4,432 mile |  | 1,917.69 | 1.92 |
| Cars (by size) - Petrol - Medium | 20,866 mile |  | 5,952.60 | 5.95 |
| Cars (by size) - Petrol - Small | 18,390 mile |  | 4,252.89 | 4.25 |
| Public transport - Local bus (not London) | 13,535 mile |  | 2,831.46 | 2.83 |
| Public transport - National rail | 8,466 mile |  | 483.13 | 0.48 |
| Total Scope 3 |  |  | 36,348 | 36 |
| Total Scope 1, 2 & 3 |  |  | 42,193 | 42 |
| TotaltCO2e per*FTE on gross scope 1, 2 & 3 |  |  |  | 2.48 |
| TotaltCO2e per*£m Turnover on gross scope 1, 2 & 3 |  |  |  | 0 |

Adjustments

*Notes: For 01 April 2024 to 31 March 2025 the number of Full-time equivalent employees (FTE) was 17 and the Turnover was GBP UNKNOWN

Energy efficiency measures taken

- Portland Consulting Engineers has continued to place greater emphasis on remote communications with clients.  Preferring to use the conferencing technology available within the office as opposed to site visits.

Energy efficiency planned

Portland Consulting Engineers plans to…

## Notes about methodology:

- Portland Consulting Engineers has adopted an operational control approach to establishing the boundary. The methodology adopted in line with the Greenhouse Gas Protocol1 and the BEIS Environmental Reporting Guidelines2. The calculations were completed on the SmartCarbonTM Calculator3 using the UK Government emissions factors4.

- CO2e is the universal unit of measurement to indicate the global warming potential (GWP) of Greenhouse Gases (GHGs), expressed in terms of the GWP of one unit of carbon dioxide. There are seven main GHGs that contribute to climate change, as covered by the Kyoto Protocol: carbon dioxide (CO2), methane (CH4), nitrous oxide (N2O), hydrofluorocarbons (HFCs), perfluorocarbons (PFCs), sulphur hexafluoride (SF6) and nitrogen trifluoride (NF3). Different activities emit different gases. Using CO₂e allows all greenhouse gases to be measured on a like-for-like basis.

- For National grid electricity consumption, THE ORGANISATION has included factors for the transmission and distribution of electricity (T&D) losses, which occur between the power station and site(s). The emissions from T&D has been accounted for in Scope 3. As with other Scope 3 impacts, reporting T&D is voluntary but is recommended standard practice by UK Government2.

Estimations:

- Client to add detail.

## Exclusions:

- Client to add detail.

Definitions:

Carbon footprint - The total set of greenhouse gas emissions (GHG) caused directly and indirectly by an individual event, organisation, or product expressed as Carbon Dioxide Equivalent (CO2e). (Source: Greenhouse Gas Protocol).

Scope 1 (direct emissions) emissions are those from activities owned or controlled by your organisation. Examples of Scope 1 emissions include emissions from combustion in owned or controlled boilers, furnaces and vehicles; and emissions from chemical production in owned or controlled process equipment.

Scope 2 (energy indirect) emissions are those released into the atmosphere that are associated with your consumption of purchased electricity, heat, steam and cooling. These indirect emissions are a consequence of your organisation’s energy use, but occur at sources you do not own or control.

Scope 3 (other indirect) emissions are a consequence of your actions that occur at sources you do not own or control and are not classed as Scope 2 emissions. Examples of Scope 3 emissions are business travel by means not owned or controlled by your organisation, waste disposal, materials or fuels your organisation purchases. Deciding if emissions from a vehicle, office or factory that you use are Scope 1 or Scope 3 may depend on how you define your operational boundaries. Scope 3 emissions can be from activities that are upstream or downstream of your organisation. More information on Scope 3 and other aspects of reporting can be found in the Greenhouse Gas Protocol Corporate Standard.

References:

- The GHG Protocol Corporate Accounting and Reporting Standard. Revised Edition (2015) World Resource Institute and World Business Council for Sustainable Development.

- Environmental Reporting Guidelines: Including streamlined energy and carbon reporting guidance (March 2019) UK Government Department for Business, Environment and Industrial Strategy.

- SmartCarbon Calculator: https://www.smartcarboncalculator.com/

- Greenhouse gas reporting: conversion factors  - Full set (for advanced users). More at this link:

https://www.gov.uk/government/collections/government-conversion-factors-for-company-reporting